This document describes the procedures and information necessary to use

the Special Character Interrupt features of the DigiBoard XAPCM232 DOS Driver

(V 1.9.1).

The driver has been enhanced to handle Special Character Interrupts (SCI) from

the Digiboard. The modifications provide for a Special Character Flag (SCF) and

a Special Character Count (SCC) for each channel supported by the driver.

Interrupt 14h functions 19h (Enable/Disable Special Character interrupts) and

and 1ah (Get pointer to Special Character Flag/Count) have been added to

facilitate easy use of these features.

The Special Character Flag :

The SCF is a byte location for each channel and has two possible values :

00h - No Special Characters in the channel's

receive buffer.

FFh - Special Character(s) in the channel's

receive buffer.

The driver will set the SCF to FFh on each SCI and reset it to 00h only after

the last received Special Character has been retrieved from the buffer, or the

buffer has been flushed.

The Special Character Count :

The SCC is a word location for each channel and specifies the number of characters

in the receive buffer, up to and including the last received Special Character.

( Note that the SCC is NOT a count of the Special Characters in the buffer, but

a count of all characters in the buffer, up to and including the last received

Special Character.) The SCC is valid only when the SCF is set to FFh. The SCC

is calculated each time a SCI occurs and is decremented each time a character

is read from the channel's receive buffer. The SCC is set to 0 if the buffer is

flushed or the last Special Character is read from the buffer. The SCC permits

easy use of INT 14h function 0Fh, the read string function, which requires that

the user specify the number of bytes to read.

Enabling SC Interrupts :

As mentioned earlier, each channel must have it's SCI's enabled. To do this,

you can use the DOS driver INT 14h function 19h (Enable/Disable Special

Character Interrupts). This function has the following usage :

FNCT 19H - ENABLE / DISABLE  SPECIAL CHARACTER INTERRUPT

ENTRY     AH : 19H

BL : 0    - DISABLE SPECIAL CHARACTER INTERRUPT

BH : 0FFH - ENABLE SPECIAL CHARACTER INTERRUPT

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER

RETURNS   AH : 0    - FUNCTION GOOD

AL : 0FFH - FUNCTION FAILED

Enabling the SCI for each channel will allow the SCFs and SCCs to provide

accurate information.

Locating the SCFs and the SCCs :

The SCFs and the SCCs are located in the driver's memory space and can be

located using the driver's Interrupt 14h function 1Ah. The function usage

is shown below :

FNCT 1AH - GET POINTER TO SPECIAL CHARACTER FLAG/COUNTER

ENTRY     AH : 1AH

BX : 0 - RETURN POINTER TO SPECIAL CHARACTER FLAG (BYTE)

BX : 1 - RETURN POINTER TO SPECIAL CHARACTER COUNTER (WORD)

DX : ANY CHANNEL NUMBER SUPPORTED BY THE DRIVER.

RETURNS   ES : BX - POINTS TO THE FLAG OR COUNTER.

NOTE : SPECIAL CHARACTER INTERRUPTS MUST BE ENABLED FOR THE

FLAGS AND COUNTERS TO PROVIDE VALID INFORMATION.

FLAG  = FFh if a Special Character is in the receive buffer.

= 0   No Special Character in buffer; count is invalid.

COUNT = The number of characters in the receive buffer, up to

and including the last received Special Character. This

count is not valid if the Special Character Flag is 0.

Typically, the application would get pointers to a channel's flag and count,

and monitor the flag until it changes from 0 to 0FFh. At this time, the

application could issue an interrupt 14h function 0FH call (read string) to

read the number of characters specfied by the count value.