SPECIFYING NON-STANDARD ADDRESSES AND INTERRUPTS

------------------------------------------------

The complete format for invoking the GHOST program is:

GHOST [Port] [Baud Rate] <Cxxxx:ixx>

where:  [Port] = COM1-8                       *REQUIRED*

or DIG1-32/XX (XX=COM dropfile translation.)

[Baud] = 300-38400                    *REQUIRED*

Cxxxx  = I/O address (4 digits, hex)  (optional)

ixx    = interrupt (decimal, 2-15)    (optional)

Examples:

GHOST COM1 9600            (ISA Com Port 1)

GHOST COM8 19200 C5228:i3  (Microchannel COM8)

GHOST DIG8/8 9600          (Intelligent DigiBoard)

The DIG format is used with intelligent DigiBoards.  The first number

specifies the DigiBoard port, where the number after the slash represents

the COM port number passed to your doors program via the drop file.