NOTE:  This file is formatted for printing.  To print on LaserJet

compatible laser printers, simply type from DOS:

copy ripkit.doc prn

For other laser printers or for dot matrix printers, you may need

to paginate the document or otherwise reformat it or add printer

codes in order to print it directly.

eSoft's TBBS RIP Kit

Version 2.3

A Series of Sample Files and Instructions

for Adding Remote Imaging Protocol Support to TBBS 2.3

Copyright (C) 1995 Philip L. Becker, Ltd.

PLEASE NOTE:  This product is copyrighted, including all programs,

sample files and documentation.  However, you are permitted to

make and distribute copies of the product as desired, for any

reason, and include modifications if you wish, provided you do not

represent any copies you make as the original eSoft-provided TBBS

RIP Kit.  This official TBBS RIP Kit is available only from eSoft,

Inc., and eSoft will provide technical support only for copies

obtained directly from eSoft.

RIP, RIPscrip, RIPaint, RIPterm and Remote Imaging Protocol are

trademarks of TeleGrafix Communications, Inc.

+----------------------------------------------------------+

|                    IMPORTANT WARNING!                    |

|                                                          |

| eSoft does provide technical support for the TBBS RIP    |

| Kit, when the kit is purchased from eSoft.               |

|                                                          |

| Please understand that we cannot provide you with        |

| training or support regarding the RIPscrip protocol      |

| itself, or with products that are not those of eSoft,    |

| such as RIPaint, RIPterm, etc.  You will need to obtain  |

| assistance with the RIPscrip protocol itself from other  |

| sources, and you will need to obtain technical support   |

| with non-eSoft products from the makers of those         |

| products.                                                |

|                                                          |

| We also cannot assist you with changes you make or want  |

| to make to the RIP.LNG language file.  There are complex |

| and not always obvious interactions between various      |

| elements of the language file; make changes to the       |

| language file at your own risk.                          |

+----------------------------------------------------------+

INTRODUCTION

RIP, the Remote Imaging Protocol, has taken the BBS community by

storm since it was demonstrated at ONE BBSCON '92 in Denver.

Many BBS software vendors and terminal software vendors have

announced RIP support.

Our feeling is that RIP is not in its current form an ideal

solution for providing online graphical interfaces for a BBS, any

more than ANSI was an ideal solution for character based display

enhancement.  RIP does provide a number of very interesting (and in

some cases quite elegant) features that help to "jazz up" a BBS

quickly and easily with graphical elements.  In its current form,

however, RIP 1.x does not provide many of the features we've heard

our customers ask us for.  For example, there is no direct

support for menu bars, pull-down menus, dialog boxes, scrolling

pointer select lists, input fields, pop-up menus, scalable

windows, etc. -- user interface elements that have been

popularized by the Microsoft Windows operating environment.

RIP is also not as easy to work with as many people might wish.

Although it's trivially simple to create screens for RIP,

implementing RIP in things like TBBS' language file, in TDBS

applications, or other places where a simple direct file display

is not possible or not desirable, it's not quite so easy to work

with.

Despite its drawbacks, RIP offers a number of terrific features.

It has support for a mouse, graphical icons, "hot" mouse regions,

definable variables, a variety of vector imaging techniques,

screen save and restore, and a number of others.

The demand for RIP has exploded, and TBBS sysops have been

anxious to have a packaged solution to offer a RIP interface to

their callers.

WHAT'S INVOLVED IN IMPLEMENTING RIP ON TBBS 2.3

RIP implementation involves two general areas:

1)  The TBBS language file.

2)  TBBS menu displays.

The purpose of TBBS 2.3 RIP support, therefore, is to:

1)  Provide a TBBS language file with RIP codes in it.

2)  Provide RIP sample menus which can be modified

without knowledge of the RIPscrip protocol.

3)  Provide internal TBBS RIP support to make bulding

RIP menus easier.

4)  Document and discuss your various options for RIP.

The language file is needed in order to make the internal

displays of TBBS graphical in nature for RIP users.  The menu

displays of course are needed to make your menus graphical for

RIP users.

INSTALLING RIP SUPPORT IN TBBS 2.3

RIP support can be installed using the eSoft 2.3 INSTALL program

located on disk one of your TBBS software.  Here is a list of your

options and what steps the INSTALL program will perform for each:

If you are installing TBBS for the first time, you can select

the RIPkit installation and install the RIP Sample Menus as

well.  At that point, you are up-and-running with a

RIP-capable system in minutes.  There are the steps that the

INSTALL program follows for you in this case:

1. Install the RIP.LNG 2.3 language file.

2. Create a LANGUAGE.BBS file.

3. Enable multi-language support in TBBS (by modifying

the RUNBBS.BAT file).

4. Create a RIP.CFG configuration file so that

RIP menu commands are executed properly, and

the initial RIP poll is enabled.

5. Ensure that CEDIT (CONFIG.CTL) has the necessary

language settings for RIP operation.

6. Install the RIP Sample Menus.

If you already have an existing TBBS system, but would like

to add RIP emulation to it with RIPkit 2.3, you can select

the Install RIPkit option from the INSTALL program menu. The

RIP.LNG language file will be installed, but the RIP sample

menus will not.  Therefore, RIP callers will see a RIP

version of all menu commands, but they will still see your

existing non-RIP menus.  Here are the steps that the INSTALL

program follows for you in this case:

1. Install the RIP.LNG 2.3 language file.

2. Create a LANGUAGE.BBS file.

3. Enable multi-language support in TBBS (by modifying

the RUNBBS.BAT file).

4. Create a RIP.CFG configuration file so that

RIP menu commands are executed properly, and

the initial RIP poll is enabled.

5. Ensure that CEDIT (CONFIG.CTL) has the necessary

language settings for RIP operation.

6. Create a top level menu which redirects the RIP

caller to your existing non-RIP top level menu

(MENU0000.CTL).

CONVERTING FROM TBBS 2.2 RIP KIT INSTALLATIONS

If you already have a TBBS system, and already have RIPkit

version 1.0 or 1.1 installed, then you can select the RIPkit

upgrade from the INSTALL program.  The new 2.3 RIP language

file would replace your existing RIP.LNG file (ANY

CUSTOMIZATIONS YOU HAVE MADE ON YOUR OWN TO YOUR EXISTING

RIP.LNG FILE WILL BE LOST).  No changes are made to your menu

structure whatsoever.  These steps are followed by INSTALL

for you:

1. Replace the existing RIP.LNG file with the

RIP.LNG 2.3 language file.

2. Replace the existing RIPOM.CFG file with the

RIP.CFG configuration file so that RIP menu

commands are executed properly, and the initial

RIP poll is enabled.

3. Ensure that CEDIT (CONFIG.CTL) has the necessary

language settings for RIP operation.

The installation program will perform all steps shown

automatically.

WHAT TO DO AFTER RUNNING INSTALL

After you have run the installation program, you will need to

create RIP menus for your TBBS system.  As mentioned above, you

can select from one of three ways to get this accomplished:

1) Make existing menus RIP compatible

2) Make new menus using supplied SDL samples

3) Make menu displays using RIPaint and {filename} method

Which one of these ways you pick depends on your specific needs

and preferences.  These three options are discussed later.

ICONS FOR THE eSoft RIP KIT

Some of the internal displays within our RIP language file

require the user to have RIP icons resident on their computer,

not unlike most RIP implementations on bulletin board systems.

We have created and supplied you with the icons we use with the

RIP Kit.  These files all have a .ICN file extension.

You may wish to provide these icons for download on your BBS so

that your users can obtain and install them in their RIP terminal

program.  If your installed the RIP Sample Menus, a routine is

included to automatically download these icons file to a caller

if they do not have your "flag file".  The flag file is a dummy

text file with a name of "E_<date>.ICN".  The <date> portion

is defined within the PATHS.SDL file, which can be found in your

MENUS directory.  This date should only change when you have

altered or added to the available icons included with the RIPkit.

See the PATHS.SDL file for more information.

IMPORTANT NOTE FOR USING TBBS MENUS WITH RIP

RIPkit 2.3 combined with TBBS 2.3 is RIP-aware without the use of

"stub" menus or the Type 204 menu command as required by previous

versions of RIPkit (1.0 and 1.1).  It is important to note, howver,

that TBBS 2.3 internally implements fully backward compatible Type

204 commands which will allow your RIPkit 1.0 and 1.1 menus to

function properly as-is, but these commands are no longer required.

THE RIP.CFG FILE

The internal TBBS RIP.CFG file is fully backward compatible with

the RIPkit 1.0 and 1.1 RIPOM.CFG file.  In addition it contains

new configuration commands to implement the extended RIP support

found in TBBS 2.3.

The new configuration command RIP_LANG: defines which language

letters are RIP format files.  The syntax is as follows:

RIP_LANG: a[, b[,c...]]

Where a, b, c represent the letters of the language files defined

by LANGUAGES.BBS which are RIP format.  Examples:

RIP_LANG: B

This defines that language B is RIP.

RIP_LANG: B, D

This defines that languages B and D are both RIP.

In TBBS 2.3, internal RIP support can be invoked automatically

for any user which has selected a language defined here as RIP.

The RIP_POLL: command has been added to force a poll of the

caller's ability to handle RIP and/or ANSI prior to the opening

language selection prompt.  The syntax is:

RIP_POLL: NO|YES|AUTO=c

NO =  don't do a RIP poll (default)

YES =  do a RIP poll, record the answer, but take no other action.

AUTO=c do a RIP poll, record the answer, and if the answer is

that the caller can do RIP, immediately switch the

language to language "c".

Example:

RIP_POLL: AUTO=B

Means poll for RIP, and if the caller can do RIP, immediately

switch to language B.

Note:  If AUTO is selected, and the language is switched by a

positive RIP poll, then any opening language selection menu prompt

is skipped.  The opening language prompt will only occur if the

poll is negative AND CEDIT is set not to default language

selection to language "A".

The answer to a RIP poll may be queried through the use of the new

insertion parameter %CANRIP% which will return a "X" if the caller

is RIP capable, and a "." if he is not.  This allows testing for

RIP ability in QAL or other TBBS operations later in the call.

If you have also set CEDIT to request an ANSI poll, then either

RIP_POLL: YES or RIP_POLL: AUTO=c will poll for both RIP and ANSI

at the opening menu.  The normal position for an ANSI poll will be

suppressed in this case, as the poll will have already occurred as

the very first action after connect.  From this poll the two

insertion parameters %CANANSI% and %CANRIP% will be set to reflect

the capabilities of the caller.

TBBS uses the language letter to determine whether RIP processing

is required or not.  If a user selects a language listed in RIP_LANG:

then RIP processing is assumed.  Any other language letter means

that RIP processing is not to be done.

The new command RIPSEQn_TYPES: has been added to allow the

definition of which TBBS internal commands require RIP prefix strings.

TBBS will automatically provide these prefixes when a RIP language

(as defined by the RIP_LANG: command) is selected, eliminating the

need for most TYPE=204 prefix string commands when building RIP

menus.  The syntax of this command is:

RIPSEQn_TYPES: n[,n [,n...]]

Where n is the decimal menu command type(s) which should have the

RIPSEQn string as a prefix.  You may also use a value of "OM" for

n to indicate that all option module calls should be prefixed with

the indicated RIP sequence, or "MENU" for n to indicate that all

menus should be prefaced with the indicated sequence.

Examples:

RIPSEQ1_TYPES: 1,2,4,20,34,46,48

RIPSEQ2_TYPES: 32,33, OM, MENU

These commands indicate that menu command types 1, 2, 4, 20, 34,

46 and 48 should be prefixed by RIPSEQ1: string whenever they are

selected if a RIP language is active.  Menu command types 32, 33,

and all option modules (200 - 255) should be prefixed with RIPSEQ2

strings.

Note:  With this definition, no further special menu handling is

required to assure proper handling of internal TBBS commands when

using RIP.  All ANSI based commands will be properly displayed

to a RIP user, and fully integrated with whatever level of true RIP

support you add to your system.

The TYPE=204 command has been extended as follows:

TYPE=204 OPT DATA=0 <type> <opt data>

This command will execute the menu option specified ONLY if the

user is currently in a RIP language.  If the user is not in a RIP

language, this command will be a no operation (NOP).  This allows

menu commands to be conditionally executed based on whether the

caller has RIP active or not.  Example:

TYPE=204 OPT DATA=0 5 RIP1

This command will do a TYPE=5, OPT DATA=RIP1 command only if the

caller is in a RIP language.

The RIP.CFG configuration file lists which TBBS menu commands

use which RIPscrip entrance sequences.  It is not necessary to

adjust these settings, and changes to them are made at your

own risk.

The purpose of these sequences is to simply issue a RIP code AFTER

the user selects a function from the TBBS menu, but BEFORE the

function itself gets executed.  This "send AFTER selection"

behavior is not one which TBBS supports by itself, hence the need

for entrance sequence definitions to perform this behavior.

RIP sequence #1 is defined as follows:

RIPSEQ1: ^A|*|w05002C0N11|1^[0000$STW$$MKILL$

This sequence is used for the following command types:

Type 1       File Display

Type 2       Multi-file display

Type 4       File areas

Type 14      File maintenance

Type 20      Keyword search

Type 22      Last 128 callers listing

Type 34      Raw download

Type 46      DIR download

Type 48      DIR delete

These command types are configured as sequence #1 as follows:

RIPSEQ1_TYPES: 1,2,4,14,20,22,34,46,48

RIP sequence #2 is defined as follows:

RIPSEQ2: ^A|*|w0000270N02|1K

This sequence is used for the following command types:

Type 32      Questionnaire entry

Type 33      Questionnaire display

ALL option modules

(Type 200 through Type 255)

These command types are configured as sequence #2 as follows:

RIPSEQ2_TYPES: 32,33,OM

RIP sequence #3 is defined as follows:

RIPSEQ3: ^A|1^[0000$SAVEALL$

(RIP sequence for type 7's so that exit from the FSE is handled

properly when message reading was not taking place)

This sequence is used for the following command types:

Type 7       Send message

This command type is configured as sequence #3 as follows:

RIPSEQ3_TYPES: 7

RIP sequence #4 is defined as follows:

RIPSEQ4: ^A|*|w0000270N02|1K

(RIP sequence to precede non-RIP menus while in RIP mode)

This sequence is used for the following command types:

Menus

(Displayed before every menu)

This command type is configured as sequence #4 as follows:

RIPSEQ4_TYPES: MENU

RIP MENUS

If you installed TBBS for the first time, and allowed the INSTALL

program to implement the RIP sample menus, your system will

already support RIPscrip emulation.  The RIP sample menu system

is contained within the MENU_RIP.SDL file.  Comments in that

SDL file will help you through any minor modifications you wish

to make.

If you had a TBBS 2.2 installation and have just upgraded to TBBS 2.3

and RIPkit 2.3, and did not allow the INSTALL program to implement

the RIP sample menus, then your system has been configured for RIP

emulation but will use your existing non-RIP menus.  If you wish

your menus to use RIP as well, you will need to create the RIP

menus yourself, and modify the top level of your RIP menu

structure (if RIP is the only additional langauge installed on

your system, then that menu is 00LB) so that a RIP caller is sent

to your RIP main menu, rather than MENU0000.CTL.

If you had a TBBS -and- RIPkit 1.0/1.1 installation and have just

upgraded to TBBS 2.3 and RIPkit 2.3, then the new RIP.LNG has

replaced the old, but no menu changes have been made.  Your RIP

installation should not be adversely affected.  Because TBBS 2.3

no longer requires the use of Type 204 commands to execute, you

may convert your RIP caller menu entries to the standard format

if you wish.  Example:

Old RIP Method:     Type=204 Optdata=2 32 REGISTER

TBBS 2.3 Method:     Type=32 Optdata=REGISTER

RIP MENUS -- MAKE MENU DISPLAYS USING RIPAINT AND {filename} METHOD

If you wish to design your own RIP menu design (rather than using

the RIP samples provided with RIPkit 2.3), you will need a copy of

RIPaint, not supplied with our RIP Kit. It is available from its

authors:

TeleGrafix Communications, Inc.

16458 Bolsa Chica #15

Huntington Beach, CA  92649

(714) 379-2131 Voice

(714) 379-2132 Fax

Essentially, you create menu displays using the RIPaint product,

and link them to your TBBS menus using the {filename} method shown

in Chapter 9 of your TBBS, in the section titled "Colorizing Your

Menus (ANSI Menu Displays)."  Using the access flag you selected

during the installation process, you show the RIP file to your

callers conditional to that flag.

For this method to work well, you must have all of your menu text

contained in external files and referenced with the { } method.

If you have individual text menu entries, you must make sure that

the text doesn't display to your RIP callers.  In other words,

only RIP files or text containing RIP codes should be displayed

to RIP callers; non-RIP text may corrupt RIP displays if allowed

to be passed to RIP callers.  You will need to ensure that this

does not occur.

HINTS AND KINKS FOR USING RIP

Here are a number of comments and suggestions regarding the use

of RIP with TBBS:

1)  Keep RIP displays to RIP callers, and non-RIP displays to

non-RIP callers.  Sending RIP to a non-RIP user will simply mean

the user sees meaningless junk characters.  Sending non-RIP

displays to RIP users can often result in corrupted RIP displays.

2)  If you want to display a RIP scene to someone, you need to

make sure that you insert a clear screen (form feed character,

ASCII 12) every 10 to 20 lines in the RIP file to keep TBBS from

pausing with a "-More-" in the middle of the display (which will

corrupt the RIP display, and confuse users).  To do this, you

must first create the RIP scene using RIPaint (or another RIP

paint or draw program), and save the scene to disk.  Edit the

RIP file using a text editor.  Every 10 to 20 lines, insert a

form feed character using your editor.

NOTE: TBBS 2.3 will not pause between pages when displaying a

file within a menu (using the curled-brackets "{filename}"

method). Therefore, clear screen codes do not need to be

inserted in these files.

3)  RIP is not error corrected.  On noisy lines, RIP displays

will become corrupted unless you and your users take advantage

of error correcting modems.  There is not a way to work around

this limitation.

4)  If you want to program RIP applications in TDBS, keep in mind

that a series of RIP codes ALWAYS must end with a carriage

return.  If you use the ? or ?? output methods in TDBS, the

carriage return comes BEFORE the string that's sent, not AFTER.

Therefore, you must add a carriage return to the end of a

sequence, either by adding:

+chr(13)

to the end of the string, or by using this at the end:

? ""

Here is an example of this concept in RIP:

? chr(1)+"|1M0J2I083M1400000001"

? chr(1)+"|1M0K5008641400000002"

? chr(1)+"|1M0L7I088M1400000003"

? chr(1)+"|1M0MA008B41400000004"

? ""

You should also note that we recommend beginning RIP codes in

TDBS with a Ctrl-A character, chr(1), instead of the standard

bang symbol (!).  This is a RIP synonym that allows a RIP code to

begin somewhere other than the beginning of a line, and has

proven to be much more reliable that the bang symbol.

We have also supplied a "RIP polling" demonstration in TDBS with

the RIP Kit (look in the RIPKIT sub-directory off your TBBS

directory) which you can use in your TDBS applications if desired

to automatically sense RIP compatibility.

5)  Hot-keys in TBBS can interrupt RIP screens, with sometimes

undesirable results.  If a user presses a hot-key in the middle

of a RIP menu display, TBBS will immediately process the hot-key

as usual.  Interrupting a RIP display, however, can leave some

RIP settings (such as colors, fonts, etc.) such that they will

look "odd" or inappropriate on subsequently displayed functions.

You should warn users not to use hot-keys in the middle of menu

displays to avoid this behavior.  If they use only buttons (part

of the RIP screen) with their mouse, this isn't a problem.

6)  If you use fonts in your menu designs other than the default

font in the default size (1x), you must remember to reset the

font to the default within your menus.  Otherwise, internal TBBS

functions may look "odd" or inappropriate in some cases.  To

reset, as the last element of a menu design you're creating in

RIPaint, simply select the default font in 1x size from the font

menu.  THIS MUST BE THE LAST THING YOU DO IN RIPAINT PRIOR TO

SAVING YOUR FINISHED MENU DESIGN.  As an alternative, you can

edit the .RIP file with your text editor, making the very last

item in the .RIP file the following:

!|Y00000100|#|#|#

This must be the LAST thing in the file, and must begin flush

left (on a new line).

7)  Please note that when a user executes a Type 31 to set message

preferences, they are presented with options to select from

ALWAYS, NEVER and ASK.  However, regardless of which one they

select, their mode will always be set to ASK.  This is

intentional.

8)  When constructing a menu entry to activate a Type 31 command

in RIP, you should use the /S switch as Opt Data, so that the

current settings are displayed immediately.  This makes for a proper

screen display to the user in RIP mode.